Border flown scenic piece designed to conceal the upper part of the stage and its machinery or equipment

Box set setting which encloses the acting area on three sides.  Conventionally in imitation of a room in which the fourth wall has been removed

Brace portable support for Flats

Bridge walkway above the stage used to reach stage equipment

C

Call (1) warning given at intervals to technicians and actors that they are needed on stage (2) notice of the time at which actors will be required to rehearse a particular scene

Callboard notice board on which calls and all other information relevant to the production should be posted

Cans headsets used for communication and co ordination of technical departments during a performance

Centerline imaginary line drawn from rear to front of stage and dividing it exactly in half.  Marked as CL on stage plans

Channel a circuit in the lighting or sound system

Chase a repeated sequence of changing lighting states

Check to diminish the intensity of Light or sound on stage

Cinemoid a colour medium or filter

Circuit the means by which a lantern s connected to a dimmer or patch panel

Clamp C or G clamps are attached to lights to fasten them to bars

Cleat fixing on the back of flats to allow them to be laced together (cleated) with a sash line or cleatline.  Also a metal fly raiI to which ropes are tied

Clothscene scene played before downstage drop or tabs, while a major scene change takes place

Colour call the Iist of coloured gels required for a lighting design taken from the plan of the lighting design

Colour frame holder for the colour medium or filter in front of the light

Colour Medium translucent filter material placed n front of lights to give a coloured rumination

Colour wheel in lighting, a device attached to lamps which, when rotated, charges the colour medium through which the light is shown

Come down (1) instruction to actor to move towards the audience (2) instruction to lower intensity of sound or light (3) end of performance; time when curtain comes down

Comer plate triangle f plywood used to reinforce the corners of a flat

Counterweights mechanical system used for raising and lowering flown scenery

Counterweight flying the

system of flying scenery, lights etc., whereby the flown item is balanced by counterweights

Crossfade the practice of moving to a new lighting or sound effect without intervening darkness or silence: one effect fades out simultaneously with the new one’s being brought into play

Crossover (I) the device on a sound system that routes the sound of the correct pitch to the correct part of the loudspeaker, (2) the space behind the stage setting or below the stage through which actors can get from one side of the stage to the other without being seen by the audience.

Cue ( I ) verbal or physical signal for an actor to enter or speak a line (2) point at which an effect is executed or bus ness takes place

Cue light box with two lights, red and green, which warn an actor or technician to standby (red) and then do (green) whatever is required of them.  Ensures greater precision when visibility or audibility s limited

Cue sheet list of particular effects executed by one department in a production

Cue-to-cue rehearsal of technical effects n a product on with actors.  The scene is rehearsed in sections beginning with a cue for standby, and concluding when the effect is finished

Curtain calls process of actors appearing at the end of the play to receive audience applause.  Formerly the audience called actors before the curtain

Curtain speech out of character address to the audience by a cast member or participant

Curtain up (1) time at which a play begins (2) a call given to the company to warn them the performance has begun

Cut cloth vertical scenic piece cut to reveal more scenery behind it. Most common in musicals

Cutting list list of materials required for scenery and set construction together with the correct dimensions of the pieces

Cyclorama undecorated backing to a stage, usually semi-circular and creating a sense of space and height.  Often some theatres have permanent or standing cycloramas, which have actually been built. The term is always abbreviated to cyc

D

Dead (I) the point at which a piece of scenery reaches the desired position onstage (2) a redundant product on or scenic element

Decibel dB the measurement of volume of sound

Diffusion (colour) used Iike a gel but to soften and spread the beam of light rather than to colour it Also called a frost

Dim the process of decreasing the intensity of light onstage

Dimmers the apparatus whereby lights are electrically dimmed

Dip small covered hole in stage floor with electric sockets

Dock area at side or rear of stage where scenery s stored when not in use

Downstage part of stage neatest to audience

Dress circle a so known as the circle.  Area of seating above the stalls and below the balcony

Dressing items used to decorate a setting

Dress parades the final check of costumes before the first dress rehearsal.  The cast parade each of the r costumes n order before the

Director and Costume Designer so that any final alterations can be made

Drop suspended cloth flown into stage area

DSH deputy stage manager

Dutchman (US) thin piece of material used to cover the cracks between two flats

E

Elevation a working drawing usually drawn accurately and to scale, showing the side view of the set or lighting arrangement Ellipsoidal the type of reflector used in many profile spots

Entrance (1) place on a set through which the actor may appear (2) point in the script at which an actor appears

Exit ( I ) the process of leaving the stage (2) point in the script at which an actor leaves the stage

F

Fader a means of controlling the output level of a lantern (amp) or amplifier.

False Pros False proscenium construction placed behind the real theatre proscenium for decorative or practical purposes

Fit-up installation of lighting, technical equipment and scenery onstage when coming into a theatre

Flash-out system to check whether the lights are functioning proper y by putting them on one at a t me

Flat scenic unit comprised of wood or stretched cloth applied to a timber frame and supported so that it stands vertical to the stage door.  Door flats and window flats have these openings in them Masking flats are placed at the outer edges of the acting area to disguise areas of the stage from the public

Flies area above the stage in which scenery, lighting and other equipment are kept.  If whole backdrops are to be stored then he fl es should be at least twice the height of the stage opening

Floodlights also called floods.  Lights which give a  general fixed spread of light

Floorcloth painted canvas sheets placed on the stage floor to give a specific effect

Floor pocket (US) see dip

Fl . own (1) scenery or equipment which has been suspended above the stage (2) flown pieces are any scenic elements which will be made to appear or disappear from view in sight of the audience

Fly the process of bringing scenery in and out of the stage area vertically

Flying ( I ) the process of stocking the flies (2) spec al effects whereby actors are suspended by wires to create the illusion of flying

FlyFloor gallery at either side of the stage from which the flies are operated

Floats see footlights

Focusing the process of fixing the exact area to be it by each light onstage

FOH Front of house.  Any part of the theatre in front of the proscenium arch

Follow spot light directed at actor which can follow all movements

Footlights  lights set into the stage at floor level which throw strong general light into performers faces downstage

Fourth wall imaginary wall between audience and actors which completes the naturalistic room

French brace support for scenery fixed to stage

Fresnel type of spotlight with a fresnel lens which gives an even field of light with soft edges

Frontcloth see cloth

Front-of-House lights lights hung in front of the proscenium arch

Frost see diffusion

G

Gauze painted cloth screen, opaque when lit from the front, !hat becomes transparent when lit from behind Often used at front of stage to diffuse total stage picture

Gel Colour medium introduced before light to alter colour of beam

Get-in/out (US) see fit up process of bringing scenery into or taking t out of the theatre

Ghost a beam of light which inadvertenty leaks from a light an d falls where it is not wanted

Gobo ( I ) screen introduced before a stage light to give a particular image onstage (2) cut out shape that is projected

Green room general area in which cast and crew wait during performance

Grid metal frame from which al flying equipment s suspended

Groundrow raised section of scenery usual y depicting bushes rocks etc.

Grouping (US) see blocking

H

Half ha f hour ca I Warning to company given thirty-five minutes before performance

Handprop any prop handled by an actor, such as a handbag, walking stick, and umbrella

Hanging attaching flying pieces to appropriate bars

Hook clamp the device that holds a lantern onto a bar

Hot lining the method by which lanterns, bulbs and cables are checked during rigging

House (1) audience (2) in opera, the entire theatre, and by implication, the company

Impedance a term for the electrical resistance found in a/c circuits, thus affecting the ability of a cable to transmit sound as electrical                      pulses, Measured in ohms

In one (US) see Clothscene

Inset a small scene set inside a larger one

Iris a device within a lantern which allows a circular beam to be altered through a range of sizes

Iron a fireproof curtain that can be dropped downstage of the tabs in case of fire.  Today t s usual y made of solid metal and is electrically operated

K

Kill instruction to cease use of particular effect in lighting or sound

L

Ladder a ladder-shaped frame used for hanging side Lights It cannot usual y be climbed

Lamp unit of lighting equipment

Lantern see amp

Left stage left.  That part of the stage to the actors left when) he is facing toward the audience

Leg cloth suspended vertically from flies and used to mask sides of stage and small areas within it

Levels ( I ) indicate intensity or volume of light or sound (2) raised areas onstage used for acting

Limes jargon for to low spots and their operators

Line drawings (US) see technical drawing

Linnebach projector used for projecting a picture from a gel or glass slide onto the set.  Often used to give a shadow effect

Load in/out (US) see get in/out

Lose to turn off Lighting or sound, or to remove an article from the set

Luminaire international term for lighting equipment.  Not restricted to theatrics lighting

M

Marking (I) use of props or scenery, the deployment of substitutes for the real object during rehearsal (2) in singing, a

means of using the voice with reduced volume and without vocalizing extremes of register (3) any account of a role in which the full powers are not being used by the performer in order to save resources

Maroon a pyrotechnic giving the effect of a loud explosion

Mark out the system of lines and objects set on a rehearsal room floor to indicate the exact position of scenery and furniture.  Marking out is the process of doing this

Mask to hide or conceal unwanted areas or machinery also used to describe one actor obscuring another on intentionally

MD musical director

Memory memory board.  An advanced type of lighting control system where the required levels are stored electronically

Mezzanine area of seating above the orchestra and below the ha cony When a theatre has on y a single balcony, first several rows are frequent y designated the mezzanine

Mixer sound controls desk, used to m x and adjust levels of sounds from various sources

0

Offstage any backstage area not seen by the audience most specifically used to indicate the areas at the actor’s right and left

OP opposite prompt.  Stage Fight (US Stage left)

Orchestra (US) sees stalls

Out flying term for up

Overture (1) the music which begins a performance (2) a ca I to the actors and technicians that the performance s about to begin a musical work

PA system the public address or any sound amplification system

Pack a number of flats a I stored together

Pan (1) movement of Iighting from side to s de (2) used to describe water-based stage make up (pancakes) (3) term (now near y obsolete) to describe theatre sound installation

Parcan type of lantern which holds a par amp

Patch border panel a panel at which the circuits governed by individual lighting dimmers can be changed

Perch   lighting position concealed behind the proscenium

Periactus a tall, prism shaped piece of painted scenery which can be revolved to show various phases

Pipe (US) see bar

Places please (US) see beginners

Platform (US) sees rostrum

Plot (1) commonly used to describe the action of a play (2) any list of cues for effects used in the play

PM production manager

Practical any object which must do onstage the same job that it would do in Life, or any working apparatus e.g. a light switch or water tap (faucet)

Preset ( I ) used to describe any article placed in its working area before the performance commences (2) a so describes a basic lighting state that the audience sees before the act on begins

Projector (US) see floodlight

Prompt copy fully annotated copy of the play withal the production details from which the show s run each t me t is performed

Properties props Any item or article used by the actors in performance other than costume and scenery

Props skip basket or cupboard n which props are kept when not n use

Props table table in convenient offstage area on which al properties are left prior to performance and to which they should be returned when dead

Pros proscenium arch the arch, which stands between stage and auditorium.  A pros arch theatre s a conventional theatre with a proscenium arch, usually without a forestage

PS prompt side.  Conventionally meaning stage left, the term now refers only to the side of the stage in which the prompt corner will be found.  In the US the PS is generally stage right

Prompt corner desk and console at the s de of the stage from which the stage manager runs the show

Pyrotechnics any chemical effects used onstage or n wings to create Iighting or special effects

Q

Quarter back stage pre-show call given twenty minutes before curtain up (i.e. fifteen minutes before beginners)

Rail bottom or top batten of the frame of a fiat

Rake the incline of a stage floor away from the horizontal; a raked stage is higher at the upstage end than at the downstage

Readthrough early rehearsal at which the play is read without action.  Usually accompanied by discussion

Reflectors the shiny surfaces in the back of lighting equipment which help intensify the beam Rigging the means of fixing lamps to appropriate bars before lighting a production

Right stage right.  That part of the stage to the actor’s right when he is facing the audience

Risers the vertical part of a stage step

Rostrum a raised platform sometimes with a collapsible frame used for giving local prominence to certain areas onstage

Run (1) the number of scheduled performances of a work (2) abbreviated form of run through

Runners a pair of curtains parting at the centre and moving horizontally

s

Saturation rig an arrangement of lights n which the maximum number of spotlights s placed in every possible position

Scatter the light outside the ma n beam of a spot

Scrim (US) sees gauze

Seque musical term indicating that one number should go immediately into the next

Set to prepare the stage for action.  To set up is to get ready to set back is to return to the beginning of a given sequence

Shutter device in front of lamp to alter shape of beam

Single purchase counterweight flying system where the cradle travels the same distance as the fly bar’s travel.  The counterweight frame therefore occupies the full height of the side wall of the stage

Sightlines the angles of visibility from the auditorium

SM stage manager

Snap line chalk line, chalked piece of string which when stretched tight is used for making straight lines on stage

Special piece of lighting equipment whose main function is to perform a particular effect

Spiking see marking

Spill unwanted light on stage

Spot spotlight.  Light giving a small circle of light, the dimensions of which can be precisely controlled by focusing

Stagger-run run through at which the production is piecing together, aiming at fluency but allowing for corrective stops

Stalls floor level area of seating in the auditorium

Strike instruct on to remove any redundant or unnecessary object from stage

Super non-speaking actor not specifically named in the text

Swag curtains or tabs gathered together so they do not hang straight

Switchboard board from which lights are controlled

T

Tabs theatre curtains, most usually the House curtain

Tabtrack metal track on which the tabs run allowing them to open and close

Tallescope extendable ladder on wheels used in rigging and focusing lights and for minor corrections to flown pieces

Teaser short flown border used to mask scenery or equipment

Tech technical rehearsal at which all technical effects are rehearsed in the context of the whole production

Theatre in the Round act rig area with audience on all sides

Throw in lighting, the distance between a light source and the object lit

Thrust stage type of stage which projects into the auditorium so that the audience can sit on at least three sides

Tilt the vertical movement of light

Tormentor (US) sees teaser

Trap hole cut in stage and concealed by floor allowing access from below, Grave traps are usually double traps creating the illusion of a grave or pit.  Once a common part of all theatres traps are now becoming increasingly rare

Trapeze single short hung lighting bar

Treads the flat part of stage steps

Truck movable cradle upon which scenery is placed to facilitate its movement

Upstage in a proscenium or

thrust stage the area furthest away from the audience

w

Wagon (US) sees truck

Walk-through rehearsals at

which actors go through entrances, moves and exits to make clear any changes or alterations made necessary through change of cast or venue

Warning bells (US) see bar bells

Ways the maximum number of combinations of channels on a 1ighting installation

Wings  the sides of the stage concealed from the audience s view

Work-out in a dance or movement rehearsal, a vigorous session to prepare the body for specific work

Workshop any non-performing backstage area of a theatre

Workshop performance a performance in which maximum effort goes towards acting and interpretation rather than sets or costumes

Musical theatre

special glossary

Andante walking space

Allegro happily, lightly

Allargando getting broader

Coda last section of music, often in a different tempo or mood

Cadence the resolving chords in music

Largo broadly

Lento slowly

Maestoso majestically

Presto fast

Aria solo, usually reflective in

content

Duet musical number for two singers

Trio three singers

Quartet four singers

Ensemble (I) together (2) place

in which all the characters all sing together

Finale (1) the end (2) by extension, a musical sequence which ends each act, often comprising different musical material but having an overall shape MD musical director Band parts the individual copies required by each player in an orchestra and containing only the notes for their particular instrument.

BIBLIOGRAPH                            y

Listed below are a representative selection of books for each of the titles in this series.

In the United Kingdom Spotlight publish annually Contacts, a corn plete guide to the British Stage, TV, Screen and Radio (7 Leicester Place, London WC2.  Tel: 071 437 7631)

In the United States the Theatre Communications Group Inc.

(TCG) (355 Lexington Avenue

New York, NY 100 1 7. Tel

: 212

697 5230) has a publications department which publishes not only plays and books but also a monthly magazine of news and features called American Theatre. It also publishes an employmen bulletin for the performing arts t called Art SEARCH.

Bentley, Eric Theory  of the  Modem Stage, London, 1968

Brook, Peter The Empty Space, London,1985

Brown, John R Drama and the Theatre, London, 1971

Hogget L Chris Stage and the Theatre, London, 1971

Oren Parker, Smith, W L, Harvey

R Scene Design and Stage Lighting,

London,1979

Stanlislawski, K An Actor Prepares, London,1981

Costume and Make-up

Barton, Lucy historic Costume for the Stage, Boston, 1938

Barton, Lucy Period Patterns,

Boston, 1942

Corson, Richard Fashions in Hair, London,1985

Corson, Richard Stage Make-up, New York 1960

Cunnington, Phillis and Lucas,

Catherine Occupational Costume in

England, London, 1967

Directing a Play

Berry, Cicely Voice and the Actor, London and New York, 1974

Hagen, Uta and Frankel, Haskel

Respect for Acting, New York, 1980

Hodgson, John and Richards, Ernest Improvisation, London, 1978; New York, 1979

Nicoll, A The Development ofthe

Theatre, London and New York, 1966

Willett.  John The Theatre of8ertolt

Brecht.  London, 1983; New York, 1968

Lightig,,.d S ... d

-

Bentham, Fredrick Art of Stage

Lighting, London, 1980; New

York. 1968

Burris-Meyer, H and Mallory, V

Sound in the Theatre, New York, 1979

Moore, j E Design for Good

Acoustics, London, 196 1; New

York, 1979

Pilbrow, Richard Stage Lighting, London and New York, 1979

Reid, Francis Stage Lighting

Handbook, London, 1982; New

York, 1976

Stage Design and Properties

Govier, jacquie Create Your Own

Stage Props, London and New

York, 1984

Leaci-oft, Richard and Helen

Theatre & Playhouse, London, 1984

Molinari, Cesare Theatre Through the Ages, London and New York, 1975

Oren Parker, W L Smith, Harvey,

R Scene Design and Stage Lighting

London and New York, 1979

Stage Management and Theatre Administravon

Baker, Hendrik Stage Management and Theatre @ (3rd Edition), London and New York, 1981

Bond, David Stage Management A Gentle Art, London 1991

Crampton, Esme A Handbook of the Theatre, London and New York, 1980

Gruver, Bert The Stage Monager’s Handbook, New York, 1972

Reid, Francis The Staging Handbook, New York, 1978

SUPPLIERS

AND

STOCKISTS

Listed below are a representa-tive selection of suppliers and stockists.

UNITED KINGDOM

Costume, Props and Make-Up

Angels and Bermans

40 Camden Street

London NW I ODX

Tel:	0 1 71 387 0999

Fax:	0 1 71 383 5603

Bapty and Co. Ltd (weapon hire) 703 Harrow Road

London NW IO SNY

Tel:	0 1 81 969 6671

Fax:	0 1 8 1 960 1106

Borovick Fabrics Ltd (theatrical) 16 Berwick Street

London WI V 3 RG

Tel:	0 1 71 43 7 2180/0520

Fax:	0 1 71 494 4646

Bristol Old Vic Hire

Units I and 2

Hayward Road Industrial Estate Staple Hill

Bristol BS 1 6 4NT Tel: 0 1 17 970 1026

Brodie and Middleton (dyes, canvas, metal powders and other paints)

68 Drury Lane

London WC2B SSP

Tel- 0 1 71 836 3289

Fax:O ,7 ,497 8425

125